# CMPG 325 – Computer Networks

| Student | Molewa, MS |
| --- | --- |
| Student number | 43628362 |
| Project ID | CMPG325-2026-084 |
| Client ID | CLI-084 |
| Assigned organisation | Ramothibe Labour Law Consultants (Kimberley) |
| Industry | Legal Services |

# 1. Client Requirements

## 1.1 Client Background

Ramothibe Labour Law Consultants is a legal services firm based in Kimberley, and my job on this project is to design them a network that gives their staff and systems a reliable and dependable connectivity. My design has to stay specific to this client.

I'll be working within the requirements, constraints and change request set out in the CMPG 325 brief, building and testing everything in Cisco Packet Tracer. Beyond just functioning in the moment the network also needs room to grow, since the firm expects to expand over the next few years.

## 1.2 Current Networking Need

The client needs network that lets different categories of traffic move around reliably without getting interrupted, staff need to get their work done, management needs its own separation, and voice traffic needs a place too.

Because my assigned challenge is VLAN Trunking over IEEE 802.1Q, the topology has to include more than one switch, and I need to show trunk links carrying VLAN traffic.

On top of that, the client has just raised a change request: management wants to move to VoIP handsets (CR11). Voice traffic has to be accommodated alongside everything else already running, so I've built in a separate logical path for it rather than adding it on as an after.

## 1.3 Functional Requirements

- Provide reliable connectivity between the required client devices and network services.

- Use the assigned IPv4 addressing block: 172.30.54.0/23.

- Implement VLANs to logically separate relevant categories of network traffic.

- Implement IEEE 802.1Q trunking between switches so that multiple VLANs can traverse the relevant inter switch links.

- Provide appropriate Layer 3 connectivity between VLANs where required by the organisation.

- Provide a dedicated logical network for voice traffic to accommodate the CR11 VoIP change request.

- Provide appropriate network management addressing for network infrastructure devices.

- Provide sufficient addressing capacity for the expected growth of the organisation.

- Ensure that the final design can be implemented and tested in Cisco Packet Tracer.

- Ensure that the completed network can demonstrate successful connectivity and operation through appropriate testing.

## 1.4 VLAN Requirements

| VLAN | Name | Purpose |
| --- | --- | --- |
| 10 | STAFF-DATA | Normal staff or user data traffic |
| 20 | MANAGEMENT | Management users and management-related devices |
| 30 | VOICE | VoIP handsets and voice traffic |
| 40 | SERVERS | Servers and network services |
| 99 | NETWORK-MGMT | Management of network infrastructure |

## 1.5 VLAN Trunking Requirement

My assigned challenge is VLAN Trunking 802.1Q across switches, which means the topology needs more than one switch, connected by trunk links carrying tagged traffic for multiple VLANs at once.

Staff-Data, Management and Voice are the VLANs that need to move between switches, and I will verify all of this later in Packet Tracer.

## 1.6 VoIP Requirement CR11

CR11 says management is adopting VoIP handsets, and voice traffic needs to be accommodated on top of the existing design but not by building a second separate network for it.

My solution is a dedicated Voice VLAN, so voice traffic stays logically apart from ordinary data traffic while still reaching the services it needs

## 1.7 Scalability Requirement

The brief expects roughly 40% user growth within three years, so I can't just design for today's headcount — the addressing plan and the topology both need headroom built in from the start.

I've used VLSM to size each subnet to what it actually needs rather than handing out identical blocks everywhere, and I've deliberately left address space unused within the /23 for whatever comes next.

That way, if the firm grows the way the brief expects, I'm not stuck redoing the whole addressing scheme from scratch.

## 1.8 IP Addressing Requirement

Everything here needs to be inside the client's assigned block(172.30.54.0/23). My addressing plan emphasises on:

- Network addresses

- Subnet masks or prefix lengths

- Usable host ranges

- Broadcast addresses

- Default gateways

- VLAN associations

- Infrastructure addresses

- Potential DHCP ranges

- Address space reserved for future growth

## 1.9 Network Device Requirements

To make this work, I need at minimum a router or another Layer 3 device, several switches, end-user PCs, IP phones, at least one server, and the connections to connect everything together.

## 1.10 Connectivity Requirements

Once it's built, the network has to work: connectivity within a VLAN, between VLANs, across trunk links, out to network services, and from voice devices too, all the way through to correct gateway behaviour and stable operation after configuration changes.

## 1.11 Design Constraints

- The network must remain specific to the assigned client, Ramothibe Labour Law Consultants (Kimberley).

- The assigned addressing block must be used: 172.30.54.0/23.

- The assigned networking challenge must be implemented: 802.1Q VLAN trunking across switches.

- The change request must be accommodated: CR11 VoIP handsets for management.

- The network must account for approximately 40% user growth over three years.

- The implementation must be produced and tested in Cisco Packet Tracer.

- The final solution must provide evidence of successful operation.

## 1.12 Security and Traffic Separation Considerations

Legal services firms handle sensitive material, so keeping traffic logically separated matters here more than it might elsewhere. Splitting staff, management, voice, server and infrastructure-management traffic into their own VLANs isn't just being tidy, it's a reasonable  step toward tighter security controls later, like access control policies when the time comes.

## 1.13 Availability and Reliability

I want this network to hold up for ordinary day-to-day use. Using multiple switches lets me demonstrate the trunking requirement. Once im done building it, I will test it to confirm it behaves the way I expect.

# 2. Physical Topology

## 2.1 Overview

## 2.2 Device List

| Device | Quantity | Purpose |
| --- | --- | --- |
| Router (Cisco 1941) — R1 | 1 | Inter-VLAN routing or the default gateway for all VLANs |
| Switch (2960) — SW1 | 1 | Core/Distribution switch |
| Switch (2960) — SW2, SW3 | 2 | Connect end devices |
| PC | 4 | End-user devices |
| IP Phone | 2 | VoIP handsets |
| Server | 1 | Network services |

## 2.3 Link Descriptions

| Link | Type / Description |
| --- | --- |
| R1 G0/0 ↔ SW1 G0/1 | 802.1Q Trunk |
| SW1 G0/2 ↔ SW2 G0/1 | 802.1Q Trunk |
| SW1 Fa0/24 ↔ SW3 G0/1 | 802.1Q Trunk |
| SW2 ↔ End Devices (PC1, PC2, IP Phone 1) | Access Links |
| SW3 ↔ End Devices (PC3, PC4, IP Phone 2, Server1) | Access Links |

## 2.4 Cable Types

- Router ↔ Switch (802.1Q trunk): Copper Straight-Through

- Switch ↔ Switch (trunk): Copper Straight-Through

- Switch ↔ End Devices: Copper Straight-Through

## 2.5 Topology Explanation

- R1 provides Layer 3 inter-VLAN routing for the whole network.

- SW1 acts as the core/distribution switch and is the central point for all VLANs.

- SW2 and SW3 are access switches that connect end devices.

- All links between switches and to the router are configured as 802.1Q trunks to carry multiple VLANs.

- PCs and servers connect over access ports tied to their own data VLAN. IP phone ports carry a data VLAN plus Voice VLAN 30 (switchport voice vlan 30) at the same time, so both the connected user's data traffic and their phone's voice traffic get handled properly.

## 2.6 Design Benefits

- Scalable: new access switches can be added and connected to SW1 using trunk links.

- Efficient: multiple VLANs share the same physical links between switches.

- Manageable: clear separation between core/distribution and access layers.

- Supports client requirements: VLAN segmentation, 802.1Q trunking, VoIP (CR11), and scalable growth.

## 3.1 Logical Topology

My logical topology mirrors the physical one: VLANs split staff data, management, voice, server and network-management traffic apart, and 802.1Q trunks carry all of it across the same physical links between switches.

R1 handles inter-VLAN routing, so traffic can move between VLANs where it's supposed to while staying logically separated everywhere else. Trunking runs between SW1 and both access switches, carrying VLANs 10, 20, 30, 40 and 99 network-wide. I've also set VLAN 99 (NETWORK-MGMT) as the native VLAN on those trunks, partly because it gives me somewhere for infrastructure-management addressing to live, and partly because it means I'm not leaving VLAN 1 as native, which is asking for trouble security-wise.

## 3.2 VLAN and Connectivity Design

- VLAN 10 will be used for normal staff data traffic.

- VLAN 20 will be used for management devices and users.

- VLAN 30 will be dedicated to VoIP traffic to accommodate CR11.

- VLAN 40 will be used for servers and network services.

- VLAN 99 will be used for managing the network infrastructure.

- End-user devices will connect to switch access ports assigned to the appropriate VLAN.

- Links between the router and switches, and between switches, will operate as 802.1Q trunk links.

- R1 will provide inter-VLAN routing using a router-on-a-stick design.

- The design allows additional users and access switches to be added to support the required 40% future growth.

## 3.4 Design Justification

Splitting traffic this way keeps things manageable and directly answers my assigned 802.1Q challenge. The dedicated Voice VLAN covers CR11, and because the VLAN structure and addressing both have room to grow, the network shouldn't need a redesign every time the firm adds people.

R1 handles all the inter-VLAN routing through router-on-a-stick, with one sub-interface per VLAN on the R1–SW1 trunk, which is what actually satisfies Functional Requirement 5.

# 4. IP Addressing Plan

## 4.1 Assigned Addressing Block

The client's been assigned 172.30.54.0/23, mask 255.255.254.0 which gives 512 addresses across 172.30.54.0 to 172.30.55.

## 4.2 Proposed Subnet Allocation

| VLAN | Name | Network | Prefix | Mask | Usable Hosts | Gateway |
| --- | --- | --- | --- | --- | --- | --- |
| 10 | STAFF-DATA | 172.30.54.0 | /25 | 255.255.255.128 | 126 | 172.30.54.1 |
| 30 | VOICE | 172.30.54.128 | /26 | 255.255.255.192 | 62 | 172.30.54.129 |
| 20 | MANAGEMENT | 172.30.54.192 | /27 | 255.255.255.224 | 30 | 172.30.54.193 |
| 40 | SERVERS | 172.30.54.224 | /28 | 255.255.255.240 | 14 | 172.30.54.225 |
| 99 | NETWORK-MGMT | 172.30.55.0 | /28 | 255.255.255.240 | 14 | 172.30.55.1 |

## 4.3 Example Device Addressing

| Device | VLAN | IP Address | Assignment |
| --- | --- | --- | --- |
| R1 – VLAN 10 Gateway | 10 | 172.30.54.1 | Static |
| R1 – VLAN 30 Gateway | 30 | 172.30.54.129 | Static |
| R1 – VLAN 20 Gateway | 20 | 172.30.54.193 | Static |
| R1 – VLAN 40 Gateway | 40 | 172.30.54.225 | Static |
| R1 – VLAN 99 Gateway | 99 | 172.30.55.1 | Static |
| SW1 Management | 99 | 172.30.55.2 | Static |
| SW2 Management | 99 | 172.30.55.3 | Static |
| SW3 Management | 99 | 172.30.55.4 | Static |
| Server1 (DNS, file/print) | 40 | 172.30.54.226 | Static |

## 4.5 DHCP Addressing Plan

| VLAN | DHCP Range | Notes |
| --- | --- | --- |
| 10 | 172.30.54.10 – 172.30.54.126 | Staff data devices |
| 30 | 172.30.54.140 – 172.30.54.190 | Voice/VoIP handsets |
| 20 | 172.30.54.200 – 172.30.54.222 | Management devices |
| 40 | Primarily static | Servers |
| 99 | Primarily static | Network infrastructure |

## 4.6 40% Growth Requirement

| Network | Initial Assumption | 40% Growth | Required Capacity |
| --- | --- | --- | --- |
| Staff | 60 users/devices | 24 | 84 |
| Management | 15 users/devices | 6 | 21 |
| Voice | 20 phones | 8 | 28 |
| Servers | 8 devices/services | 4 | 12 |

## 4.7 Address Space Reserved for Future Expansion

There's a lot more room in this /23 than what my current VLANs uses. 172.30.54.240 – 172.30.54.255 (16 addresses) plus the whole of 172.30.55.16 – 172.30.55.255 (240 addresses) are still free, which is roughly half the original block. I'm holding that back for whatever comes up later, such as:

- Additional staff VLANs

- Additional access switches

- Additional servers

- Wireless networks

- Additional VoIP devices

- Guest networks

- Future departments

- Additional network services

Basically, I didn't want to burn through the client's whole address block on day one.

## 4.8 IP Addressing Design Justification

The sizing here isn't arbitrary, I went largest-to-smallest specifically so every subnet would land on a clean boundary. Staff gets the biggest share because it has the most endpoints; Voice gets a /26 so there's real headroom for CR11 and whatever comes after it; Management gets a /27; and Servers and Network-Management both get /28s, since neither needs much more than a handful of addresses.

# 5. Initial GitHub Repository